Problem 1 [25 pts]: Using PVA analysis

Consider the mechanism drawn below.  The goal is to algebraically determine the velocity, position, and acceleration at point C using PVA analysis. Assume the following:

O2= (0,0),	O4 = (27.5,0),

A=(7.5,10),	B=(19.5,15),	C=(3.5,14),

ω4=1 rad/s,	α4= -2rad/s2

Hint: This problem may look like one you completed when we studied instant centers, which could be useful for performing a sanity check on your answers

- Write out the vector loop equations for the position, velocity, and acceleration of C

- Compute the lengths of O2A, O2O4, AB, BO4, and AC

- Compute the angles ϑ2, ϑ3, ϑ4, ϑ5

- Compute the angular velocities ω 2, ω 3, ω 4, ω 5

- Compute the angular accelerations α 2, α 3, α 4, α 5

- State the linear velocity and acceleration of point C

Show all work for full points

Select one of the following options:

- My answer was created by a Gen AI algorithm, and I have not modified it

- My answer was created by a Gen AI algorithm, and I have made some minor changes.

- My answer was created by a Gen AI algorithm, and I have made major changes.

- My answer was created solely by myself.

- If I used Gen AI, I used ___ (name of program).