Deadline and submission information on Canvas

Total: 25 points

Problem 1 [12 pts]: Short answers on gears

  - (2 pt) In complete sentences define the Diametral Pitch, Pitch Point, and Pitch Diameter.

  - (2 pt) Explain why straight triangular teeth are not common in gears and describe the construction of the most common tooth profile.

  - (3 pt) What does the pressure angle represent on a gear? What are the advantages of a higher pressure angle? What are the advantages of a lower pressure angle?

  - (3 pts) Select 3 different types of gears and give an example of when that gear type would be used.

  - (2 pt) What is the function of an idler gear, and how does it affect the angular velocity ratio in a gear train?

Problem 2 [13 pts]: Designing a gear set

You are given the following set of spur gears, all having a pressure angle of 20°.

Gear # 1: N= 36, d= 9.6mm(N=number of teeth; d=pitch diameter)

Gear # 2: N= 75, d= 25mm

Gear # 3: N=36, d=12mm

Gear # 4: N= 90, d= 15mm

Gear # 5: N= 36, d= 6mm

Gear # 6: N= 75, d= 20mm

Gear # 7: N=180, d=60mm

Gear # 8: N= 150, d= 40mm

  - (3 pt) Group the gears into sets that can mesh together. For the following two problems, do not try to combine two gears that do not mesh.

  - (5 pt) Select some of these gears to make a simple gear train with velocity ratio of 0.5, where the input and output have different directions. Make a sketch of the gear train, indicating the input and output gears, angular velocity direction and the gear # for each gear. You may reuse gears, but your solution should use as few gears as possible.

  - (5 pt) Now design a compound gear train where the input and output shafts have the same direction of rotation and a velocity ratio of 10 using the same set of spur gears given at the start of this problem. Make a sketch of the gear train, indicating the input and output gears, angular velocity direction and the gear # for each gear. You may reuse gears, but your solution should use as few gears as possible.

Select one of the following options:

- My answer was created by a Gen AI algorithm, and I have not modified it

- My answer was created by a Gen AI algorithm, and I have made some minor changes.

- My answer was created by a Gen AI algorithm, and I have made major changes.

- My answer was created solely by myself.

- If I used Gen AI, I used ___ (name of program).