PVA Analysis Appendix

Step 1: PVA analysis:

Figure 1: Example of labelled leg mechanism defining links, relevant angles, vector loops, and positive orientations for position vector and angular position.

Known: All link lengths, angleθ2, angleθ9, angleθ10, are fixed. angleθ1 is known. Link4 and link5 are rigid parts, link6 and link8 are rigid parts, the angle between the two linkages don’t change.

Loop1 (starting from ground O2, and going clockwise)

0= R3⃑-R4⃑-R1⃑-R2⃑= L3eiθ3-L4eiθ4-L1eiθ1-L2eiθ2

Known: θ1, θ2; Unknown: θ3, θ4.

Loop2 (concluding node1, node6, node7, node5)

0= R6⃑-R7⃑-R10⃑-R1-6⃑= L6eiθ6-L7eiθ7-L10eiθ10-L1-6eiθ1-6

Known: θ10, θ1-6; Unknown: θ6, θ7.

For the position:

Loop1: Real: L3coscos θ3 -L4coscos θ4 -L1coscos θ1 -L2coscos θ2=0

Imaginary: L3sinsin θ3 -L4sinsin θ4 -L1sinsin θ1 -L2sinsin θ2=0

Loop2: Real: L6coscos θ6 -L7coscos θ7 -L10coscos θ10 -L1-6coscos θ1-6=0

Imaginary: L6sinsin θ6 -L7sinsin θ7 -L10sinsin θ10 -L1-6sinsin θ1-6=0

4 equations, to solve the 4 unknowns.

For the velocity:

Known: The angular velocity of the link1 and link2, through the fixed angle, we can know the angleθ1-6, angleθ3-4 and angleθ6-P

Unknown: The angular velocity of the link3 and link4

Loop1: Real: -L3ω3sinsin θ3 +L4ω4sinsin θ4 +L1ω1sinsin θ1 +L2ω2sinsin θ2 =0

Imaginary: L3ω3coscos θ3 +L4ω4coscos θ4 +L1ω1coscos θ1 +L2ω2coscos θ2 =0

Loop1: Real: -L6ω6sinsin θ6 +L7ω7sinsin θ7 +L10ω10sinsin θ10 +L1-6ω1-6sinsin θ1-6 =0

Imaginary: L6ω6coscos θ6 +L7ω7coscos θ7 +L10ω10coscos θ10 +L1-6ω1-6coscos θ1-6 =0

For the acceleration:

Known: The acceleration of the link1, link2, link1-6.

Unknown: The acceleration of the link3, link4, link6 and link7

Loop1: Real: -L3α3sinsin θ3 -L3ω32coscos θ3 +L4α4sinsin θ4 +L4ω42coscos θ4 +L1α1sinsin θ1 +L1ω12coscos θ1 +L2α2sinsin θ2 +L2ω22coscos θ2 =0

Imaginary: L3α3coscos θ3 -L3ω32sinsin θ3 -L4α4coscos θ4 +L4ω42sinsin θ4 -L1α1coscos θ1 +L1ω12sinsin θ1 -L2α2coscos θ2 +L2ω22sin⁡(θ2)

Loop2: Real: -L6α6sinsin θ6 -L6ω62coscos θ6 +L7α7sinsin θ7 +L7ω72coscos θ7 +L10α10sinsin θ10 +L10ω102coscos θ10 +L1-6α1-6sinsin θ1-6 +L1-6ω1-62coscos θ1-6 =0

Imaginary: L6α6coscos θ6 -L6ω62sinsin θ6 -L7α7coscos θ7 +L7ω72sinsin θ7 -L10α10coscos θ10 +L10ω102sinsin θ10 -L1-6α1-6coscos θ1-6 +L1-6ω1-62sin⁡(θ1-6)

For the foot point P

RP⃑=R1-6⃑+R6-P⃑

RP⃑=L1-6eiθ1-6+L6-Peiθ6-P

For the velocity and acceleration

VP⃑=V1-6⃑+V6-P⃑

V1-6⃑=V1-2⃑+V2-6⃑

V1-6⃑=iω1L1eiθ1+iω4(L4+L5)eiθ4

VP⃑=iω1L1eiθ1+iω4L4+L5eiθ4+iω8L8eiθ8

AP⃑=A1-6⃑+A6-P⃑

AP⃑=iα1-6L1-6eiθ1-6-ω1-62L1-6eiθ1-6+iα6-PL6-Peiθ6-P-ω6-P2L6-Peiθ6-P

Interpreting and using PVA

The image below shows a simulated movement of the robot's legs to determine the contact and lifting phases of the robot's legs.

Figure 2. Identification of lift/contact phases in mechanism plot

Lift Phase

Contact Phase

Lift Off Point

Contact Point

Lift Phase

Contact Phase

Figure 3. Identification of lift/contact phases from position diagram

As seen in the position versus time plot above, the contact and lift phases take an equal amount of time. This means that the foot is designed to be in contact with the ground for 50% of the crank, so β= 0.5. The gait sequence is shown below, with the leg numbers corresponding with the first figure in this document.

Figure 4. Gait sequence of The Klann Mechanism

The robot's stride length can be calculated by plotting the relationship between the robot's leg displacement and time.

Ls=xlift-xcontact=xt=17.5s-xt=9.5s≈10cm

Calculate the crank angular velocity based on the duty cycle and step size.

VRobot=LsβT=(Ls)(ωcrank)β(2π)

ωcrank=VRobot min*β(2π)Ls

ωcrank min=2mmin*0.5*2π0.1m=62.83radmin≈1.05s