Problem 1- Motors (12 pts)

Specifications for the motor can be found at: https://www.pololu.com/product/3228/specs

- (3 pts) We will power our motor with 8 1.2V batteries arranged in series. This will function as a single 9.6V battery. Using information from the datasheet, list your best guess for the no load speed and current, as well as stall torque and current for your motor when it is being powered by the batteries. Since the exact data for 9.6V is unavailable, you may perform a linear interpolation to identify the parameters that you need.

- (3 pts) The batteries have a capacity of 2200 mAh. Based on this information in a worst-case scenario how long should your batteries last before they need to be recharged?

- (3 pts) The motor has an internal gear ratio of 34.014:1. How does this gear train impact the power that is delivered by your motor. Mathematically defend your claim.

- (3 pts) If a project needs 70 oz-in of torque in order to function properly, what is the ideal gear ratio so that the motor delivers maximum power?

Problem 2: Using rise functions (13 pts)

The following fall function is proposed for a rise-dwell-fall-dwell motion program.

sθ=7-θβ+12πsin2πθβ

where β is the fall interval in degree, s is in mm, and θ is in degree.

a) (4 pts) Assume the cam falls from 0 to 60 degrees, dwells from 60 to 120, rises using a similar functional form from 120 to 225 and dwells from 225 to 360. Write down the piecewise function that defines the cam motion over the entire interval.

b) (8 pts) Just like in class, sketch an SVAJ plot of the position, velocity, acceleration and jerk of the function. Calculate the acceleration at θ = 0°, θ = 60°, θ = 120° and θ = 225°.

c) (1pt) Is the proposed rise function acceptable according to the Fundamental Law of Cam Design? Explain.

Select one of the following options:

- My answer was created by a Gen AI algorithm, and I have not modified it

- My answer was created by a Gen AI algorithm, and I have made some minor changes.

- My answer was created by a Gen AI algorithm, and I have made major changes.

- My answer was created solely by myself.

- If I used Gen AI, I used ___ (name of program).