Problem 1 [25 Points]: VW on a Four-Bar Mechanism

A four-bar mechanism is shown below at a specific position. A motor rotates the crank counterclockwise at 3 rad/s by providing a counterclockwise torque of 10 N.cm. Determine the angular velocity of the rocker ω4 and the external force FC by following the below steps. C is located in the middle of link 4.

Note: You may assume that the mass and moment of inertia of each link is 0.

I13

60°

60°

L1 = 18 cm

ω2 = 3 rad/s

T2 = 10 N.cm

L2 = 6 cm

L3

L4 = |O4B| = 9 cm

A

B

C

FC

30°

O2

O4

a) (7 points) Calculate the angular velocity of the rocker, ω4, and the velocity of point C, by using instant center analysis.

b) (7 points) Calculate the angular velocity of the rocker, ω4, and the velocity of point C, by using PVA analysis.

c) (11 points) Assuming frictionless joints and massless links, determine FC using the virtual work method. Indicate the correct direction of FC on the schematic after finding its value.