Problem 1 [25 pts]:

A four-bladed ceiling fan has 3-ft by 0.2-ft equally spaced rectangular blades that nominally weigh 4 lbs each. Manufacturing tolerances will cause the blade weight to vary up to plus or minus 15%. Calculate the weight of the counterweight needed at a 4-in radius to statically balance the worst-case blade assembly. The distance between the center of rotation and the inner edge of the blades is 8 in.

Problem 2 [25 pts]:

Balancing a 4-bar linkage The figure below shows a four-bar linkage and its dimensions in meters. The steel crank and rocker have uniform cross sections 40 mm wide and by 20 mm thick. The aluminum coupler is 20 mm thick. The crank O2A rotates at a constant speed of ω=50 rad/sec. Design counterweights to force-balance the linkage. Give your solution in the form of mass moment (mass x distance) and angles. If needed, use the following density for aluminum 2,700 kg/m3 and steel 8,000 kg/m3. To calculate the center of gravity for a triangle, use the formula below. Note that it is applicable to any XY axes of your choice.

C

C

A

B

C

CG

x

y

Lo2o4=5, L2=3, Lab=2, L4=4, Lac=1, Lbc=2

Use the following equations if needed without derivation:

Select one of the following options:

- My answer was created by a Gen AI algorithm, and I have not modified it

- My answer was created by a Gen AI algorithm, and I have made some minor changes.

- My answer was created by a Gen AI algorithm, and I have made major changes.

- My answer was created solely by myself.

- If I used Gen AI, I used ___ (name of program).